- Section 101100 
Visual Display Units

  - PART 1 GENERAL

    - SECTION INCLUDES

      - Glass Marker Wall.

    - REFERENCE STANDARDS

      - 16 CFR 1201 - Safety Standard for Architectural Glazing Materials; Current Edition.

      - ANSI Z97.1 - American National Standard for Safety Glazing Materials Used in Buildings - Safety Performance Specifications and Methods of Test; 2015 (Reaffirmed 2020).

    - SUBMITTALS

      - Product Data: Provide manufacturer's data on Glass Marker Wall Covering. Include Material Safety Data Sheets when applicable

      - Shop Drawings: Submit shop drawings for each type of visual display board as specified, including sectional details, trim, face and core materials, dimensions, joint locations, and any special anchor details. , special anchor details

      - Color Samples: Provide a complete set of color swatches showcasing the manufacturer's full selection of colors and finishes for Tackable Wall Covering

      - Products Samples: Two​, 10" by 10" inches [254 by 254] mm in size illustrating materials and finish, color and texture of tackable surfaces.

      - Manufacturer's printed installation instructions.

    - QUALITY ASSURANCE

      - Manufacturer Qualifications: Company specializing in manufacturing the products specified in this section with minimum 20 years documented experience

    - WARRANTY

      - Provide the manufacturer's standard three-year warranty for workmanship, confirming that, with proper usage, maintenance, and installation according to the manufacturer's guidelines, the Glass Marker Wall are guaranteed for three-year from the date of substantial completion.

  - PART 2 PRODUCTS

    - MANUFACTURERS

      - ​Visual Display Units

        - Nelson Adams NACO

        - Address: 420 S. E St. San Bernardino CA 92401

        - P: (877) 810-4080

        - Web Page: https://nelsonadamsnaco.com

    - GLASS MARKER WALL

      - ​Magnetic ​Glass Markerboard Wall: Room-size presentation surface made of multiple ​floor-to-celing​ glass panels.

        - Glass: ​​​Back-coated glass​​​, ​​​tempered, low iron​​​, ​​​1/4 inch thick (6 mm)​​​, ​​​with flat polish and square corners​​​​​​, laminated to steel backing sheet for use with magnets​​​. Coated or treated for use as ​​​dry-erase board​​​.

        - Glass Finish: ​​​As selected from manufacturer's full range​​​.

        - Steel Backing Sheet Thickness: 24 gauge, 0.0239 inch  (0.61 mm).

        - Panel Width: ​​48 inches (1220 mm)​​.

        - Height: ​​​As indicated on drawings​​​.

        - Length: ​​​As indicated on drawings​​​.

        - Accessories: Provide ​​​magnetic marker tray, magnetic marker holder, and [magnetic eraser]​​​.

    - MATERIALS

      - Glass: Provide glass-based glazing unless otherwise indicated.

        - Fully Tempered Safety Glass:  Comply with ANSI Z97.1 or 16 CFR 1201 criteria for safety glazing used in hazardous locations.

    - ACCESSORIES

      - Temporary Protective Cover: Sheet polyethylene, ​​8 mil (0.2 mm)​​ thick.

      - Magnetic Tray: Aluminum, manufacturer's standard profile, one piece full length of markerboard, molded ends, concealed fasteners, same finish as frame.

      - Magentic eraser: dry-erase board cleaning accessory with an integrated magnet that allows it to attach directly to magnetic glass board surfaces.

  - PART 3 EXECUTION

    - EXAMINATION

      - Verify that field measurements are as indicated.

      - Verify that internal wall blocking is ready to receive work and positioning dimensions are as indicated on shop drawings.

    - PREPARATION

      - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

    - INSTALLATION

      - Install boards in accordance with manufacturer's instructions.

      - Secure units level and plumb.

      - Mounting: [ Concealed Z clips ][ Stainless Steel Standoffs ]

    - CLEANING

      - Clean board surfaces in accordance with manufacturer's instructions.

      - Cover with protective cover, taped to frame.

      - Remove temporary protective cover at Date of Substantial Completion.

  - END OF SECTION